AGW TCPIP Socket Interface

New ver (2000)

updated at 25FEB200

NEW ENTRIES/Updates

The 'Y' frame reply format changed.

Added the follow new frames

1.'y' command. You can ask and receive the frames that wait for tx in a radioport queu. Those frames are all the frames from all the programs for all callsigns and unproto.

2.'X' reply command. When registering a callsign you receive back an 'X' frame reporting sucess/failure

AGW Packet Engine can serve as the L2 for your Packet Application. That Way you

can take advantage of the many unique features of AGW Packet Engine.

So far the only way to communicate with AGW Packet Engine was the DDEML.

In this document will i will describe the new way using tcpip sockets.

How to Enable tne new interface

New Versions  have an appropriate menu entry.

How to Start your application and register with AGW PAcket Engine

First of all create a stream socket and connect with AGW Packet Engine ip address

at port 8000. If your application is running at the same machine with agwpe then

as ip address use localhost (127.0.0.1).

After connecting you need to register your application's callsign if your application need to establish ax25 connections. If your application just do monitoring and sends Unproto frames ,no need for callsign registration. You can register as many as 100 different callsigns. If you register a callsign then agw packet engine accepts AX25 connections for this call.

You then ask agwpe to send you the radioport information. How many radioports are

with their describtion. After that you can create your windows or anything else.

Now you are ready. If you wish to do monitoring then you must enable monitoring.

How AGWPE communicates with your application

This is done with frames. Each frame has a header that describes the frame and

after the header are the data.

The header format is as follow

int Port; (4 bytse long )(LOWORD contins the port info,HIWORD is reserved)0 is the first port , 1 is the second port,2 is the third and so on..

int DataKind; the datakind 4 bytes long(THe LOWORD conatins the kind of this frame) The HIWORD is used as stated below

unsigned char CallFrom[10];  NULL terminated 10 bytes long including the NULL char.

unsigned char CallTo[10];  NULL terminated 10 bytes long including the NULL char.

int DataLen; The length of the data that follow. This field is 4 bytes long(32bit)

int USER (4 bytes long) reserved,undefined.

The two callsign fields may be empty. And are in plain text,like SV2AGW-12. The callsign may be less than 10 characters and the NULL terminating character is always at the end of the call not at the 10th character.

**** CAUTION********

Don’t assume that you will receive a complete frame,TCPIP may send to your program part of a frame or more than a frame so the procedure for reading data is like reading from a file.Read only what you need. Like

A complete frame is HEADER+DATA  or just HEADER with no data

1.check to see if in the stream socket there are at least HEADER bytes. If not then return

2.Examine the header and  the DataLen field

3.If the DataLen field is greater than 0 check to see if there are in the stream socket DataLen bytes.

4.If there are DataLen bytes then read exactly DataLen bytes no more, otherwise wait until DataLen bytes are available.

5.go to step 1 again till all the frames read.

Follow these steps carefully. If your application is running in the same machine with agw packet engine then the usal is that you will receive more than a frame ,if the monitor traffic is large.

What Kind Of Frames AGW Packet Engine sends to Your Program

MONITOR FRAMES

1.UNPROTO monitor frame

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='U'; The ASCII value of letter U

CallFrom= is the call from the station we heard the Packet

CallTo =is the destination call (CQ,BEACON etc)

DataLen= is the length of the data that follow

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

2.DATA That We transmit

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='T'; The ASCII value of letter T

CallFrom =The call from which we heard the frame

CallTo=is the call to whom we send the frame

DataLen is the length of the data that follow

USER is undefined.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

3.SUpervisory Frames (only Header) (RR,REJ etc)

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='S'; The ASCII value of letter S

CallFrom is empty (NULL)

CallTo is empty NULL

DataLen is the length of the data that follow

USER is undefined.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes  DataLen Bytes

4.Connected data from other stations

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='I'; The ASCII value of letter I

CallFrom is empty (NULL)

CallTo is empty NULL

DataLen is the length of the data that follow

USER is undefined.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes DataLen Bytes

Raw AX25  Frames

You can receive RAW AX25 frames if you enable this service. Those frames are all the packet valid frames received from any radioport. The frame is exactly the same as the pure AX25 frame with the follow additions.

The first byte always conatains the radioport number 0 for 1st radioport.

There is no FCS field at the end of the frame.

There is no bit stuffing.

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='K'; The ASCII value of letter K

CallFrom the callsign of the station who TX the frame(Source station)

CallTo The callsign of the destination station

DataLen is the length of the DATA field(the length of the frame

USER is undefined.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][DATA   ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes    DataLen Bytes

STATUS FRAMES(Connect, disconnect,Mheard list,and radioport info

................................................................

1.Connect( if we connect with another station)

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='C'; The ASCII value of letter C

CallFrom the callsign of the station with whom we connect

CallTo is empty (NULL)

DataLen is the length of the status message that follow

USER is undefined.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][status message   ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes    DataLen Bytes

The status messages are two

a)"*** CONNECTED To Station [CALLSIGN]" When the other station starts the connection

b)"*** CONNECTED With [CALLSIGN]" When we started the connection

2.Disconnect

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='d'; The ASCII value of letter d (lower case value)

CallFrom the callsign of the station from whom we disconnect

CallTo is empty (NULL)

DataLen is the length of the status message that follow

USER is Undefined

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][status message   ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

The status messages are two

a)"*** DISCONNECTED From Station [CALLSIGN]" when we disconnect

b)"*** DISCONNECTED RETRYOUT With [CALLSIGN]" When no answer from other station

3.How Many Outstanding frames wait for tx for a particular station

The LOWORD Port field is the port

The LOWORD DataKind field ='Y'; The ASCII value of letter Y

CallFrom our callsign or empty

CallTo is the callsign of the station for whome is the value

DataLen is the length of the data.

USER is undefined

The data frame is 4 bytes long and contains a 32bit integer that represents the outstanding frames.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][DATA]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes   4bytes

4.RadioPort information

The LOWORD Port field is always 0

The LOWORD DataKind field ='G'; The ASCII value of letter G

CallFrom  empty(NULL)

CallTo    empty(NULL)

DataLen is the length of the data that follow

USER is undefined

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes    DataLen Bytes

the data field format is as follow in plain text

howmany ports ;1st radioport description;2nd radioport;....;last radioport describtion

like

2;TNC2 on serialport 1;OE5DXL on serialport2;

We have here 2 radioports. The separator is the ';'

5.MHEARD LIST

The LOWORD Port field is the port for which the list is

The LOWORD DataKind field ='H'; the ASCII value of letter H

CallFrom  empty the call of the station in the mheard list

CallTo    empty(NULL)

DataLen is the length of the data that follow

USER is undefined

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes   DataLen Bytes

the data field format is as follow.

First there is a string (Null Terminated) with all the info like

SV2AGW Fri,22JAN1999 08:10:32   Fri,22JAN1999 22:08:03

Then follows two SYSTEMTIME structers (check windows help)

The first is the time when the station first heard and the second when last heard.

You will receive 20 separate frames for the radioport you asked the MHEARD list.

Many of them may be empty so check first the CallFrom Field to see if it is empty.

6.RadioPort Capabilities

We receive a frame like this

The LOWORD Port field is the RadioPort

The LOWORD DataKind field ='g'; The ASCII value of lower case letter g

CallFrom  empty(NULL)

CallTo    empty(NULL)

DataLen is the length of the data that follow

USER is undefined

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes   DataLen Bytes

the data field format is as follow and it is a structure

unsigned char OnairBaud;

unsigned char TrafficLevel;// if    this is 255 then the port is not in autoupdate mode

unsigned char TxDelay;

unsigned char TxTail;

unsigned char Persist;

unsigned char Slottime;

unsigned char maxframe;

unsigned char AX25Channels;  // How many connections we have

unsigned int HowManyBytes;// how many bytes are received the last 2 minutes

7.Version Number

The LOWORD Port field= is undefined

The LOWORD DataKind field ='R'; the ASCII value of letter R

CallFrom ==Undefined

CallTo =  Undefined

DataLen is the length of the data that follow

USER is undefined

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

the data field Conatains two 32bit integers. The first is the Major Version and the Second is the Minor Version. The first version that has this feauture is 98.90 (Major ver=98,minor ver=90).

8.How Many  frames wait for tx for for a RadioPort

The LOWORD Port field is the port

The LOWORD DataKind field ='y'; The ASCII value of lower case letter y

CallFrom our callsign or empty

CallTo our callsign or empty

DataLen is the length of the data.

USER is undefined

The data frame is 4 bytes long and contains a 32bit integer that represents the outstanding frames.

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][DATA]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes   4bytes

9.Reply to an 'X' command (Register a Callsign)

Port field must be 0

The LOWORD DataKind field ='X'; The ASCII value of letter X

CallFrom is our callsign

CallTo is empty(NULL)

DataLen is 1.

USER is undefined

The data frame is 1 byte long and contains the value 1 if the registrations is succesfull or 0 in failure.

1 is not ASCII value. This byte value is either 1 or 0.

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][DATA]

4bytes  4byte     10bytes  10bytes   4bytes       4bytes  1byte

10. Reply to a 'G' command. This frame returns the radioport number and description

Port field is always 0

LOWORD DataKind field ='G'. The ASCII value of letter G

CallFrom is empty (NULL)

CallTo is empty(NULL)

DataLen =The number of bytes of DATA field

USER is undefined

DATA field conatins the radioport description

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER] [DATA]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes    Datalen bytes.

The DATA field is organised as follow and is in plain ASCII.

Number of Radioports;First radioport description(Friendlyname);Second radioport description(Friendly name)...........

Number of radioports=a Decimal Value. A value of 3 means 3 radioports

Radioport description= A string that describes the radioport.

The separator between fields is the letter ';'. Just parse the whole DATA field and use as separator the ';'

DATA FROM Connected Stations

The LOWORD Port field is the port which heard the frame

The LOWORD DataKind field ='D'; The ASCII value of letter D The HIWORD contains the PID info for this frame

CallFrom is the call of the other station

CallTo is our call.  We may have more than a callsign

DataLen is the length of the data that follow

USER is undefined

the whole frame with the header is

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes   DataLen Bytes

How  your application can communicate with AGWPE

The format is the same as with the received frames a header and then the data if

there are any

int Port; (4 bytse long )(LOWORD contins the port info,HIWORD is reserved)0 is the first port , 1 is the second port,2 is the third and so on..

int DataKind; the datakind 4 bytes long(THe LOWORD conatins the kind of this frame) The HIWORD is used as stated below

unsigned char CallFrom[10];  NULL terminated 10 bytes long including the NULL char.

unsigned char CallTo[10];  NULL terminated 10 bytes long including the NULL char.

int DataLen; The length of the data that follow. This field is 4 bytes long(32bit)

int USER (4 bytes long) reserved,undefined.

EXAMPLE

This is a function that we use to send data over our stream socket to agwpe.We use it for certain frame kinds.

The port integer varible is connstructed s follow

MAKELONG(0,0);//to tx from port 0

The sme with the DataKid

MKELONG('D',0);

void  SendPacket(char *ToCall,char *str,int count,int DataKind,int port)

{

if (count==0) count=lstrlen(str);

char szTemp[3000];

MoveMemory(szTemp.&port,sizeof(int));//which port to tx

MoveMemory(szTemp+sizeof(int),&DataKind,sizeof(int));datakind here LOWORD should be ‘D’

datakind here should be ‘D’

MoveMemory(szTemp+(sizeof(int)*2),MyCall,strlen(MyCall)+1);//mycall

MoveMemory(szTemp+(sizeof(int)*2)+10,ToCall,strlen(ToCall)+1);//other station call

MoveMemory(szTemp+(sizeof(int)*2)+10+10,&count,4);//length of the data we send to other station

MoveMemory(szTemp+(sizeof(int)*4)+10+10,str,count+1);//now add the actual data after leving 4 additional bytes for USER which are reserved for the moment;

TXDATA(szTemp,count+26);//send them over our socket connection

}

Transmit DATA To Connected Stations

...........................

Port field is the port where we want the data to tx

DataKind field =MAKELONG('D',0); The ASCII value of letter D

CallFrom is our call

CallTo is the call of the other station

DataLen is the length of the data that follow

USER is undefined

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes   DataLen Bytes

EXAMPLE

Assume that we wish to send “Hello” to SV2BBO with whom we are already connected at radioport 1.

SendPacket(“SV2BBO”,”Hello\r”,6,MAKELONG(’D’,0),MAKELONG(0,0))

Transmit DATA UNPROTO

.....................

Port field is the port where we want the data to tx

DataKind field =MAKELONG('M',0); The ASCII value of letter M

CallFrom is our call

CallTo is the call of the other station

DataLen is the length of the data that follow

USER is udefined

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes       4bytes DataLen Bytes

EXAMPLE

We send a beacon text. String test contains the Beacon text and we tx the beacon over all our radioports

for (int x=0;x<HowManyPorts;x++)

{

SendPacket("BEACON",test,strlen(test),MKELONNG('M',0),MAKELONG(x,0));

}

Transmit DATA UNPROTO Via

.....................

Port field is the port where we want the data to tx

DataKind field ='V'; MAKELONG('V',0)The ASCII value of letter V

CallFrom is our call

CallTo is the call of the other station

DataLen is the length of the data that follow   in special format

USER is undefined

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

The data field contains first the via calls and then follow the actual data

the format is as follow

[   DATA  field                                 ]

[How many digis] [1stdigicall][2digicall].......

byte            10bytes     10bytes

Be carefull each digi call must be 10 char long Null terminated.If sorter then next digi must

exactly after 10 bytes as follow

e.g char str[100];

str[0]=HowManyDigis;

str+1=1digi;//null terminated

str+1+10=2digi;//null terminated

str+1+20=3digi//null terminated

..............

..............

Transmit a special frame in Raw AX25 format

.........................................

The frame must be in RAW AX25 format as should be txed by the modem(no need for bit stuffing etc)

You can use this function to tx for instance an unproto

frame with a special PID or any other frame different from normal AX25 Format.

Port field is the port where we want the data to tx

DataKind field =MAAKELONG('K',0); The ASCII value of letter K

CallFrom empty (NULL)

CallTo empty (NULL)

DataLen is the length of the data that follow

USER is Undefined

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

ASK RadioPorts Number and descriptions

..........................

Port field must be 0

DataKind field =MAKELONG('G',0); The ASCII value of letter G

CallFrom is empty (NULL)

CallTo is empty(NULL)

DataLen must be 0

USER is undefined

No data field must exists

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes

EXAMPLE

struct HEADER Header;

//build first the header

Header.port=MAKELONG(0,0);//no needed leave it 0

Header.DataKind=MAKELONG('G',0);

Header.callfrom[0]=NULL;//empty no needed

Header.callto[0]=NULL;//no needed leave it empty

Header.size=0;//no needed set it to 0

Header.USER=0;

myStreamSocket->Write((char *)&Header,sizeof(Header));//send it to AGWPE using our stream socket

Register our CallSign

......................

We need to register a call if we wish to accept connections for this callsign. That way when agwpe listend a packet for that call will notify us

Port field must be 0

DataKind field =MKELONNG('X',0); The ASCII value of letter X

CallFrom is our callsign

CallTo is empty(NULL)

DataLen must be 0

USER is undefined

No data field must exists

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4byte     10bytes  10bytes   4bytes       4bytes

int Count=0;

char szTemp[50];

MoveMemory(szTemp.&port,sizeof(int));//which port to tx

MoveMemory(szTemp+sizeof(int),&DataKind,sizeof(int));datakind here LOWORD should be ‘X’

datakind here should be ‘D’

MoveMemory(szTemp+(sizeof(int)*2),MyCall,strlen(MyCall)+1);//mycall the call we wish to register

MoveMemory(szTemp+(sizeof(int)*2)+10,ToCall,strlen(MyCall)+1);

MoveMemory(szTemp+(sizeof(int)*2)+10+10,&count,4);//length of the data we send to other station

MoveMemory(szTemp+(sizeof(int)*3)+10+10,&count,4);//User is undefined

TXDATA(szTemp,36);//send the header over our stream socket to agwpe

Unregister previously registered callsign

.........................................

Doing so AGWPE will stop responding to incoming packets for that callsign

Port field must be 0

DataKind field =MAKELONG('x',0); The ASCII value of letter x lower case

CallFrom is the callsign we wish to unregister

CallTo is empty(NULL)

DataLen must be 0

USER is undefined

No data field must exists

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

int Count=0;

char szTemp[50];

MoveMemory(szTemp.&port,sizeof(int));//which port to tx

MoveMemory(szTemp+sizeof(int),&DataKind,sizeof(int));datakind here LOWORD should be ‘x’

datakind here should be ‘D’

MoveMemory(szTemp+(sizeof(int)*2),MyCall,strlen(MyCall)+1);//mycall the call we wish to unregister

MoveMemory(szTemp+(sizeof(int)*2)+10,ToCall,strlen(MyCall)+1);

MoveMemory(szTemp+(sizeof(int)*2)+10+10,&count,4);//length of the data we send to other station

MoveMemory(szTemp+(sizeof(int)*3)+10+10,&count,4);//User is undefined

TXDATA(szTemp,36);//send the header over our stream socket to agwpe

Connect using a special PID

............................

We can use it eg for netrom connections

Port field is the port where we want the connection to be done

DataKind field =MAKELONG('c',0); The ASCII value of letter c(Lower case)

CallFrom is our call

CallTo is the call of the other station

DataLen must be 1

USER is undefined

the data field is 1 byte long and contains the PID value

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes  DataLen Bytes

Connect

.......

Port field is the port where we want the connection to be done

DataKind field =MAKELONG('C',0); The ASCII value of letter C

CallFrom is the call of the other station

CallTo is our call

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4byte     10bytes  10bytes   4bytes       4bytes

Connect VIA

............

Port field is the port where we want the connection to be done

DataKind field =MAKELONG('v',0); The ASCII value of letter v(Lower case v)

CallFrom is our call

CallTo is the call of the other station

USER is undefined

DataLen the length of the data frame where the via calls are

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER][Data         ]

4bytes  4bytes     10bytes  10bytes   4bytes   .   4bytes  DataLen Bytes

The data field is in special format and contains the via callsigns

like

[   DATA  field                                 ]

[How many digis] [1stdigicall][2digicall].......

byte            10bytes     10bytes

Be carefull each digi call must be 10 char long Null terminated.If sorter then next digi must

exactly after 10 bytes as follow

e.g char str[100];

str[0]=HowManyDigis;

str+1=1digi;//null terminated

str+1+10=2digi;//null terminated

str+1+20=3digi//null terminated

..............

..............

int count;

count=PrepareViaList(Via,ViaList);//prepare the via list in special format

if (!count) return;//no via list,something wrong return

memmove(str,ViaList,count);

count++;

MoveMemory(szTemp.&port,sizeof(int));//which port to tx

MoveMemory(szTemp+sizeof(int),&DataKind,sizeof(int));datakind here LOWORD should be ‘x’

datakind here should be ‘D’

MoveMemory(szTemp+(sizeof(int)*2),MyCall,strlen(MyCall)+1);//mycall the call we wish to unregister

MoveMemory(szTemp+(sizeof(int)*2)+10,ToCall,strlen(MyCall)+1);

MoveMemory(szTemp+(sizeof(int)*2)+10+10,&count,4);//length of the data we send to other station

MoveMemory(szTemp+(sizeof(int)*3)+10+10,&count,4);//User is undefined

MoveMemory(szTemp+(sizeof(int)*4)+10+10,str,count);//vi list formatted string

TXDATA(szTemp,count+36);

int PrepareViaList(char *InVia,char *OutVia)

{

//InVia string contains the via list like SV2AGW,SV2BBO,SV2DFK

//OutVia is the same list suitable prepared for agwpe

char *token;

char temp[15];

char HowVia=0;

token=strtok(InVia,", ");

if (token)

{

HowVia++;

strcpy(temp,token);

memmove(OutVia+1,temp,10);

}

for (;;)

{

token=strtok(NULL,", ");

if (token)

{

strcpy(temp,token);

memmove(OutVia+1+(HowVia*10),temp,10);

HowVia++;

} else break;

}//end for

OutVia[0]=HowVia;

return((HowVia*10)+1);

}

DisConnect

.......

Port field is the port where we want the Disconnection to be done

DataKind field =MAKELONG('d',0); The ASCII value of letter d(Lower case d)

CallFrom is our call

CallTo is the call of the other station

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

ASK how many outstanding frames are for a station

..................................................

Port field is the port

DataKind field =MAKELONG('Y',0); The ASCII value of letter Y

CallFrom is our call

CallTo is the call of the other station

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

ASK MHEARD list for a radioport

...............................

Port field is the port

DataKind field =MAKELONG('H',0); The ASCII value of letter H

CallFrom is empty no needed

CallTo is empty no needed

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

ASK To start receiving Monitor Frames

Sending again thios command will disable thie service.

You can start and stop this service any times you needed

Port field no needed set it to 0

DataKind field =MAKELONG('m',0); The ASCII value of lower case letter m

CallFrom is empty no needed

CallTo is empty no needed

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes

ASK To Receive RadioPort CapaBilities

Port field LOWORD is the radioPort to query

DataKind field =MAKELONG('g',0); The ASCII value of lower case letter g

CallFrom is empty no needed

CallTo is empty no needed

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

ASK To Receive AGWPE Version Info

Port field is undefined (no needed)

DataKind field =MKELONG('R',0); The ASCII value of  letter R

CallFrom is empty no needed

CallTo is empty no needed

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes

ASK To start receiving AX25 RAW Frames

Sending again thios command will disable thie service.

You can start and stop this service any times you needed

Port field no needed set it to 0

DataKind field =MAKELONG('k',0); The ASCII value of lower case letter k

CallFrom is empty no needed

CallTo is empty no needed

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes     4bytes

ASK how many outstanding frames are waiting for TX in A RadioPort Queue

..................................................

Port field is the port

DataKind field =MAKELONG('y',0); The ASCII value of letter y

CallFrom is undefined or mycall

CallTo is undefined or mycall

DataLen must be 0

USER is undefined

No data field must be present

the whole frame with the header is

[ HEADER                                   ]

[port][DataKind][CallFrom][CallTo ][DataLen][USER]

4bytes  4bytes     10bytes  10bytes   4bytes      4bytes